DA-ES1104D

Architectural and Engineering Specifications

Version 1.3

(Jan. 24. 2022)

Table of Contents

1	Manufacturer	3

2	General	3

2.1	Product Description	3

2.2	General Specification	3

2.3	Hardware Specification	3

2.4	Performance	4

2.5	Technical Specification	4

2.5.1	Front Panel – LEDs	4

2.5.2	RAID Option Switch Settings	4

2.6	Environmental Specification	5

2.7	Electrical Specification	5

2.8	Mechanical Specification	5

Version History	6

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electronic Safety and Security

Level1 28 05 00 – Common Work Results for Electronic Safety and Security

Level2 28 05 19 – Storage Appliances for Electronic Safety and Security

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Republic of Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DA-ES1104D is an eSATA Storage designed and manufactured by IDIS. This eSATA storage is a 19-inch, rack-mount Serial ATA external storage device. It supports RAID (levels 0, 1+0, and 5) for improved data stability and performance. This product is available to input up to 4 HDDs, and supports 3.0Gbps transfer rate (SATA II). It supports the maximized space and capacity relative to cost.

General Specification

- eSATA storage shall be equipped the drive which sized as 3.5” (inches).

- eSATA storage shall support SATA II drives up to 4 with 3Gbps transfer rate.

- eSATA storage shall support enhanced data stability and performance with various levels of RAID (0, 1+0, and 5).

- eSATA storage shall show the system status via front 6 LEDs: Power, D1 to D4 (HDD/RAID mode LED), and eSATA.

- eSATA storage shall support controlling RAID mode via RAID option switch on the rear panel.

- eSATA storage shall support easy scalability for the HDD space.

- Using the mute button, eSATA storage shall support turning off the audio warnings.

- eSATA storage shall support SATA to SATA port multiplier.

Hardware Specification

- Interface: eSATAx1

- Number of Drives: SATA IIx4

- Drive Size: 3.5”

- Drive Type: SATA II

- Supported Device : IR-300A

Performance

- Type of Rate: SATA II (3Gbps)

- RAID: 0, 1+0, 5

- Max Drive Capacity: Refer to 'HDD Product Compatibility' menu on idisglobal.com

Technical Specification

Front Panel – LEDs

- PWR (Power LED): Lit when the system is operating.

- D1 to D4 (HDD/RAID mode LED)

- eSATA (eSATA LED): Blue LED is lit when the unit is connected to a NVR.

- Setup Button: Initializes RAID or mutes audio warnings. (available to be locked using RAID option switch on the rear panel)

- System status indicated by audio warnings can be identified by the frequency of beeps per second.

  - Once per second: FAN error has occurred.

  - Twice per second: RAID rebuilding has started.

  - Three times per second: HDD error has occurred.

  - Five times per second: RAID initialization is ready.

RAID Option Switch Settings

- RAID Type 1 / RAID Type 2

| RAID Type1 | RAID Type2 | Mode |
| --- | --- | --- |
| OFF (Down) | OFF (Down) | SATA Port Multiplier |
| OFF (Down) | ON (Up) | RAID 1+0 |
| ON (Up) | OFF (Down) | RAID 0 |
| ON (Up) | ON (Up) | RAID 5 |

- Display

| Option Switch | Display |
| --- | --- |
| ON (Up) | RAID Mode |
| ONFF (Down) | HDD Status |

- RAID Mode

| LED D1 | LED D2 | LED D3 | LED D4 | RAID Mode |
| --- | --- | --- | --- | --- |
| Green | OFF | OFF | Red | SATA Port Multiplier |
| Green | OFF | Green | Red | RAID 1+0 |
| Green | Green | OFF | Red | RAID 0 |
| Green | Green | Green | Red | RAID 5 |

Environmental Specification

- Operating Temperature: 0°C ~ 40°C (32°F ~ 104°F)

- Operating Humidity: 0% to 90% (Non-condensing)

Electrical Specification

- Power Source: 100V~240VAC, 0.6~0.3A, 50/60Hz

- Power Consumption: Max. 40W

- Regulatory Approvals: FCC, CE

Mechanical Specification

- Dimensions (W x H x D): 480mm x 44mm x 346mm (18.9” x 1.7” x 13.6”)

- Unit Weight: 6.4kg (14.11 lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | Sep. 10. 2020 | Initial Release |
| 1.3 | TS Team | Jan. 24. 2022 | Modified Operating Temperature |
| 1.4 | TS Team | Mar. 17. 2022 | Modified ‘Max Drive Capacity’ |