[image: image1.png]


[image: image2.png]IDIS






DC-M4211WRX (2.8 / 3.3mm)
Architectural and Engineering Specifications
Version 1.0
(May. 10, 2024)
PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES
PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors
2.1.0 General
2.1.1 Product Description
DC-M4211WRX is an Outdoor Dome type IP Camera designed and manufactured by IDIS. This camera provides Full HD (1920 x 1080) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with IR LED, Fixed-Focal lens, True Day/Night, PoE (IEEE 802.3af Class 3), microSD/SDHC/SDXC card backup, Vandal-proof bullet enclosure design, IP67(IK10) rated and provides continuous operation in subzero temperature.
2.1.2 General Specification
1. The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor.
2. The IP camera shall be equipped with 
- 2.8mm fixed-focal lens, F2.0
- 3.3mm fixed-focal lens, F2.0
3. The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).
4. The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).
5. The IP camera shall have IR LED looming 10m.

6. The IP camera shall support Fixed-Iris.

7.   The IP camera shall be Vandal-Proof and IP rating 67 (IK10) complied dome enclosure design

8. The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 2
9. The IP camera shall have on board microSD/SDHC/SDXC card backup storage slot as a safeguard against data loss during network interruptions.
10. Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.
11. The IP camera shall deliver maximum video resolution of 1920 x 1080 at rates up to 30ips (images per second).
12. The IP camera shall provide direct network connection using H.265, H.264 and MJPEG compression. 
13. The IP camera shall support Triple streams 

14. The IP camera shall conform to the ONVIF Profile S Ver.2.4.0 standard. 

15. The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in. 

16. The IP camera shall have Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS), and configurable user authority levels for greater security.
17. The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.
18. The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.
19. The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm and Trip Zone.
2.1.3 Protocol Specification: DirectIP 2.0

1.  The IP camera shall have DirectIP 2.0 mode.

2.  DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

3.  DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

4.  DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

5.  DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

6.  DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

7.  DirectIP 2.0 protocol shall provide Quadruple streams.

8.  DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

2.2.0 Technical Specification
2.2.1 Video Specification
1. Image Sensor: 1/2.8” CMOS

2. Maximum Resolution: 1920 x 1080
3. Scanning Mode: Progressive Scan
4. Lens Type: 
- Fixed-focal (f=2.8, F2.0)
- Fixed-focal (f=3.3, F2.0)
5. Iris Control: Fixed Iris
6. Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

A. 2.8mm : 116.6º(H), 61.45º(V), 127.3º(D)
B. 3.3mm : 101.3º(H), 54.5º(V), 119.7º(D)
7. Pan/Tilt Range: Pan: -5° ~ 5°, Tilt: 20° ~ 75°, Rotate: -90° ~ 90°
8. Minimum Illumination:
A. COLOR : 0.1 lux @ F2.2
B. B/W: 0 lux 

9. S/N Ratio: more than 45 dB

10. Maximum Frame Rate: 
A. 30fps : 1920 x 1080(WDR)

11. Video Resolution:
A. 16:9 : 1920 x 1080, 1280 x 720, 640 x 360, 352 x 240
B. 4:3 : 1440 x 1080, 960 x 720, 480 x 360, 272 x 240
12. Video Compression : H.265, H.264, MJPEG.
13. Video Compression Level: 4 levels: Basic, Standard, High, Very High

14. Multi-Video Streaming: Triple streams

15. Dynamic Range: 120 dB
16. True Day & Night: Yes (ICR)

17. Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm and Trip Zone
18. Analog Video Output: 1 CVBS
2.2.2 Audio Specification
1. Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law 

2. Audio Input / Output: 1(Internal MIC) / 0
2.2.3 Network Specification
1. Port: RJ-45, 10/100Base-T, 1 port

2. Network Protocols: DirectIP 2.0 Protocol, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP

3.   Security: Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS)  Remote Access Client: DirectIP NVR Connection, IDIS Web, IDIS Mobile, IDIS Solution Suite
4.   Recording Session Buffer (NLTSrec): Up to 60MB
5.   Edge Storage: micro SD/SDHC/SDXC, Smart Failover (Up to 256GB)
2.2.4 Security Specification
1. Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS)
2.   Maximum User Access: 10 sessions for (Admin, Watch, etc.), 1 session for Recording, 1 session for Searching the recorded data in SD card
2.3.0 Environmental Specification
1. Operating Temperature: -30°C ~ +55°C (-22°F ~ +122°F)
2. Operating Humidity: 0% to 90% 
3. Vandal-proof Enclosure: Yes, Outdoor Dome Camera
4. Outdoor Ready: IK10 / IP67 supports
2.4.0 Electrical Specification
1. Power Source: PoE(IEEE 802.3af class 3)

2. Power Consumption: 8.9W
3. Regulatory Approvals: FCC, CE, MIL-STD-810G, EN50155(M12 model only)
2.5.0 Mechanical Specification
1. Dimensions (Ø x H x D): Ø104 x 49.44 x 122.17mm
2. Unit Weight: 0.36kg(0.79lb)
Version History
	Version
	Writer
	Revision Date
	Remarks

	1.0
	Robin Park
	May. 10, 2024
	Initial release





