DC-T4236HRX

# Architectural and Engineering Specifications

Version 1.3

(Sep. 2, 2022)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

## Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

## General

### Product Description

DC-T4236HRX is a Bullet type IP Camera designed and manufactured by IDIS. This camera provides Full HD (1920x1080) resolution at 30ips (images per second) with H.265/H.264/M-JPEG compression. This camera is equipped with Motorized Vari-focal lens, True Day/Night, PoE (IEEE 802.3af Class 3), IR LED, Audio I/O, Alarm I/O, Vandal-proof bullet enclosure design, IP67 rated and heater provide for continuous operation in subzero temperature.

### General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 2.8mm - 12mm Motorized Vari-focal lens, F1.4 - 2.8.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance. The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have Wide Dynamic Range compensation (Digital WDR) for improved video quality in high-contrast situations (120dB).

- The IP camera shall be equipped with 6ea Infrared LED with range up to 30m (98.4ft.).

- The IP camera shall utilize configurable 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be Vandal-Proof, IP rating 67 and heater.

- IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3 to supply power to the camera over the network and 12VDC input.

- The IP camera shall have built-in heater for continued use in subzero temperature conditions.

- The IP camera shall have video out feature (NTSC/PAL).

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 1920x1080 at rates up to 30ips

(Images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and M-JPEG compression.

- The IP camera shall support Quadruple Streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF ver.16.12.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have IP filtering, HTTPS, SSL, IEEE 802.1X, and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Motion Detection, Active Tampering Alarm and Trip Zone.

### Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and M-JPEG compression.

## Technical Specification

### Video Specification

- Image Sensor: 1/2.8" CMOS

- Maximum Resolution: 1920 x 1080

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal (f= 2.8 - 12mm, F1.4 - 2.8)

- IRIS Control: DC Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide : 98º(H), 51º(V), 116º(D)

  - Tele : 33º(H), 19º(V), 38º(D)

- Minimum Illumination:

  - COLOR : 0.1 lux @ F1.4

  - B/W : 0 lux (IR LED ON)

- S/N Ratio: More than 45dB

- Maximum Frame Rate: 30ips @ 1920x1080

- Video Resolution: 1920x1080, 1280x720, 640x360, 352x240

- Video Compression : H.265, H.264, M-JPEG

- Video Compression Level: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs): 30m (98.4ft.) (6ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone

### Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law.

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: On/Off

### Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, Upnp, SNMPv2

- Streaming Mode: Quadruple streaming

### Security Specification

- SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering, HTTPS

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

### Alarm and Event Specification

- Alarm Input / Output: 1/1

  - 'ALARM IN : 1 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - ALARM OUT : 1 TTL open collector, 30mA @ 5 VDC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

- Trigger Events: Motion Detection, Alarm in, Audio Detection, Tampering, Trip Zone

- Event Notification: Remote Software, Email (with Image)

## Environmental Specification

- Operating Temperature: -40°C ~ +55°C (-40°F ~ +131°F)

* Starting up at above -20°C (-4°F)

- Operating Humidity: 0% ~ 90%

- Vandal-proof Enclosure: IK10

- Outdoor Ready: IP67, Heater

## Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3af class 3)

- Power Consumption:

  - 12V=, 0.78A, 9.3W

  - PoE, IEEE 802.3af (Class 3), 8.5W

- Regulatory Approvals: FCC, CE, KC

## Mechanical Specification

- Dimensions (Ø x H): Ø77mm x 244.1mm (Ø3" x 9.61")

- Unit Weight: 0.84 kg (1.85 lb)

# Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.3 | TS Team | Sep. 2, 2022 | Spec Update |
| 1.2 | TS Team | Aug. 20, 2021 | Spec Update |
| 1.1 | TS Team | Feb. 22, 2021 | Spec Update |
| 1.0 | TS Team | Feb. 18, 2020 | Initial Release |